OBSALL Apps

(Observation Application)

Documentation (ver. 1)

Climate DT Team

April 2024

CONTENTS

CHAPTER 1: INTRODUCTION	4

1.1.	Overview of OBSALL	4

1.2.	Purpose	4

1.3.	Key Features	4

1.4.	Contributing	4

CHAPTER 2: INSTALLATION	4

2.1.	Prerequisites	4

2.2.	Installation on Lumi HPC	5

2.2.1. Cloning/ Copying	5

2.2.2. Setting environment	5

2.2.3. Executing application	6

2.2.4. Requesting modelled data	7

CHAPTER 3: MAIN COMPONENTS/ PARTS OF OBSALL	7

3.1.	Part SYNOP (ground-based/surface observations)	7

3.2.	Monitoring for SYNOP Part	8

3.2.1. Rank histograms for all synop stations for 00, 06, 12, and 18 UTCs	8

3.2.2. Standard plots for each synop station	9

3.2.3. Summary rank histograms for all synop stations	11

3.3.	Part TEMP (radiosounding observations)	14

3.4.	Monitoring for TEMP Part	15

3.4.1. Rank histograms for all radiosounding stations for 00 and 12 UTCs	16

3.4.2. Standard plots for each radiosounding station	17

3.4.3. Summary rank histograms for all radiosounding stations	18

3.5.	Part AMSUA-A (satellite observations)	21

3.6.	Monitoring for AMSU-A Part	22

3.6.1. Rank histograms for all areas for daily mean	23

3.6.2. Standard plots for each area	24

3.6.3. Summary rank histograms for all areas	26

CHAPTER 4: CONTRIBUTING TO OBSALL	29

4.1.	Reporting bugs	29

4.2.	Suggesting features	29

4.3.	Contributing to code	29

4.4.	Contributing to documentation	29

CHAPTER 5: TROUBLESHOOTING AND FAQ	30

CHAPTER 6: REFERENCES AND ACKNOWLEDGMENTS	30

6.1.	Citing OBSALL	30

6.2.	Acknowledgments	30

6.3.	Especial thanks	30

# CHAPTER 1: INTRODUCTION

## Overview of OBSALL

The OBSALL Apps is the Observation Application. It has been developed for evaluation of climate models, known as Digital Twins of the Earth. It is processing and analyzing large volumes of observation and modelled data. It has 3 main components (or Parts) such as SYNOP (for ground-based/ surface observations), TEMP/ RADSOUND (for upper are sounding observations), and AMSU-A/ SATELLITE (satellite-based observations). Important note, observational data with only free and unrestricted license have been collected and utilized for development, testing, and implementation purposes in Observation DataBase (ODB) format.

## Purpose

The purpose of the OBSALL Apps is (i) to pre-process modeled data from DestinE climate model experiments; (ii) to extract observations (from ODB) - from synoptic (ground-based/ surface observations) and radiosounding (upper air sounding) stations, and satellites; (iii) to extract modeled data at observation times and locations, applying appropriate observation operator, and ODB added with model simulation values; and (iv) to calculate/ produce relevant statistics for monitoring including illustrative material.

Important note, the end result is the augmented ODB, which can be used for: (i) on-line monitoring of the simulation in near-real time, and (ii) posterior analysis of the ClimateDT models quality.

## Key Features

- Pre-processing (with operators of cdo, Climate Data Operators, software) Global State Vector (GSV) extracted modeled data (over latitude-longitude domain) for selected meteorological variables into hourly time-slots/slices (00...23 UTCs);

- Extracting (using sql with odp_api software) selected meteorological variables at hourly time-slices for observations (from ODB) from: (i) ground-based synoptic stations at fixed geographical latitude-longitude locations on surface; (ii) radiosounding stations and satellites at fixed latitude-longitude locations on surface and on multiple pressure levels;

- Extracting and applying appropriate observation operator (using cdo operators - for synop; & expecting/using polytope - for radiosounding and satellite) to modeled data for the same time-slices for same selected meteorological variables into corresponding locations/points of (synop, radiosounding, satellite) observations, with end result - augmented ODB;

- Calculating/ producing relevant statistics for monitoring such as quantile rank histogram statistics and plots.

## Contributing

OBSALL Apps is developed under the European Union Contract DE_340_CSC - Destination Earth Programme Climate Adaptation Digital Twin (Climate DT). Contributions to the project are welcome and can be made through the OBSALL repository at GitLab hosted by BSC.

Please, see and follow guidelines in Chapter “CONTRIBUTING TO OBSALL”.

# CHAPTER 2: INSTALLATION

## Prerequisites

Before installing OBSALL Apps, ensure that you have the following:

• Access to Lumi HPC for the DestinE project; so, you need to be registered with user account.

• Git: OBSALL is hosted on GitLab, and you will need Git to clone the repository.

• Access to GitLab hosted by BSC (Barcelona Supercomputing Center); so, you need to be registered with user account. Contact Albert Vilamiro (albert.vilamiro@bsc.es) to authorize registration and project you would like to join. Once your request is validated, an email will be sent with further instructions to follow, and login at https://earth.bsc.es/gitlab/users/sign_in.

• Access to repository for OBSALL Apps; so, you need to contact Albert Vila (albert.vila@bsc.es) with CC to Pablo Ortega (pablo.ortega@bsc.es) requesting access to the "obsall” repository of the de340 project.

## Installation on Lumi HPC

### 2.2.1. Cloning/ Copying

- Clone/ copy the OBSALL Apps ("obsall" repository) to your local directory via Git using:

cd your_local_directory

git clone https://earth.bsc.es/gitlab/digital-twins/de_340/obsall.git

cd obsall

git checkout newbranch

This will clone the "newbranch"-branch of the "obsall" repository.

OR you may copy tar-file containing “obsall” repository

cd your_local_directory

cp /projappl/project_465000454/ama/ARC_OBSALL/arc_obsall_apps.tar .

tar -xvf arc_obsall_apps.tar

cd obsall

This will copy the "newbranch"-branch of the "obsall" repository.

The basic content of the “obsall” directory is:

| drwxrws--- FIGSEXAMPLES drwxrws--- .git drwxrws--- RADSOUND -rw-rw---- README.md -rw-rw---- request_radsound.yml -rw-rw---- request_synop.yml -rw-rw---- run_obsall.py -rwxrw---- run_obsall.sh drwxrws--- SATELLITE drwxrws--- SYNOP | - illustration material (figures, demo video, slides) for OBSALL Apps - contains all the log messages, author information, and other information required to rebuild any version or branch of the project - all scripts/programs/etc. for the TEMP/ RADSOUND Part - basic overall description of the Apps - request for TEMP Part (gsv extract modelled data for radionsonding obs) - request for SYNOP Part (gsv extract modelled data for synop obs) - python script to execute Apps - bash script to execute Apps - all scripts/programs/etc. for the AMSU-A/ SATELLITE Part - all scripts/programs/etc. for the SYNOP Part |
| --- | --- |

### 2.2.2. Setting environment

- Set environment (load necessary modules required for SYNOP, RADSOUND, and SATELLITE Parts) in order to run OBSALL application on LUMI: execute bash-script which is containing list of required modules (see below): ./load_modules.sh

module use /project/project_465000454/devaraju/modules/LUMI/23.03/C

module load LUMI/23.03

module load partition/C

module load PrgEnv-gnu

module load ecCodes/2.32.0-cpeCray-23.03.lua

module load odb_api/0.18.1-cpeCray-23.03.lua

module load python-climatedt/3.11.3-cpeCray-23.03.lua

module load pyfdb/0.0.2-cpeCray-23.03.lua

module load cray-hdf5/1.12.2.3

module load cray-netcdf/4.9.0.3

module load rttov/13.2

module load radsim/3.2

module load fdb/5.11.94-cpeCray-23.03.lua

module load eckit/1.25.0-cpeCray-23.03.lua

module load metkit/1.11.0-cpeCray-23.03.lua

This will load all above-listed modules.

### 2.2.3. Executing application

- Execute the OBSALL Apps by running script: python run_obsall.py

This python script will subsequently run the 3 Parts of OBSALL Apps

#!/scratch/project_465000454/devaraju/SW/LUMI-23.03/C/python-climatedt/bin/python

# OBSALL Apps (3 parts: SYNOP, TEMP, AMSU-A observations)

# Import required libraries

import sys

import subprocess

## --- Processing ground-based observations (SYNOP)

print('**********************************************************')

print('DestinE Climate Digital Twin - OBSALL Apps')

print('--- Processing ground-based observations (SYNOP)')

print('**********************************************************')

command_synop_run = "cd SYNOP; pwd; ./main_synop.sh; exit 0"

subprocess.run(command_synop_run, shell=True, check=True, executable="/bin/bash")

## --- Processing radiosounding observations (TEMP)

print('**********************************************************')

print('DestinE Climate Digital Twin - OBSALL Apps')

print('--- Processing radiosounding-based observations (TEMP)')

print('**********************************************************')

command_radsound_run = "cd RADSOUND; pwd; ./main_radsound.sh; exit 0"

subprocess.run(command_radsound_run, shell=True, check=True, executable="/bin/bash")

## Processing satellite observations (AMSU-A)

print('**********************************************************')

print('DestinE Climate Digital Twin - OBSALL Apps')

print('--- Processing satelitte-based observations (AMSU-A)')

print('**********************************************************')

command_satellite_run = "cd SATELLITE; pwd; ./main_amsua.sh; exit 0"

subprocess.run(command_satellite_run, shell=True, check=True, executable="/bin/bash")

sys.exit(0)

### 2.2.4. Requesting modelled data

- For SYNOP Part the Data request (sfc): gsv extracting modelled data at sfc-level: 2t & other available meteorological variables.

- For TEMP Part the Data request (sfc+pl): gsv extracting modelled data at (1) sfc-level: 2t, and (2) pl-level: t (at 850 hPa).

- For AMSU-A Part the Data request (sfc+pl+o2d): gsv extracting modelled data at (1) sfc-level: 2t, 2d, sp, skt,10u, 10v & z, lsm (n/a – IFS; tmp. in static grib file); (2) pl-levels: t, q, clwc, z (at 1000, 925, 850, 700, 600, 500, 400, 300, 250, 200, 150, 100, 70, 50, 30, 20, 10, 5, 1 hPa) – 19 pl-levels; and (3) o2d-level: avg_siconc – from NEMO model (or ci – from IFS, n/a now).

- Data request combined (sfc, pl, and o2d -levels): yml-file is needed for extracting selected modeled data from IFS and NEMO models at sfc-, pl-, and o2d-levels. The example (see at at https://earth.bsc.es/gitlab/digital-twins/de_340/obsall/-/tree/newbranch) is given as part of the mother_request.yml-file for extraction of all needed variables for synop (only at sfc-level), radiosounding (at both sfc- and pl-levels) and satellite (at sfc, pl-, and o2d-levels) parts of OBSALL Apps.

See details info on request of modeled data for 3 Parts of Apps:

GSVREQUEST: dataset, class, type, expver, stream, activity, resolution, generation, realization,        experiment, model, levtype, date, time, param, grid, method

OPAREQUEST: variable, stat, stat_freq, output_freq, time_step, save, checkpoint, checkpoint_filepath, save_filepath

- Container for OBSALL Apps (to be followed soon)

# CHAPTER 3: MAIN COMPONENTS/ PARTS OF OBSALL

## Part SYNOP (ground-based/surface observations)

main_synop.sh - main bash-script to run Apps for observational data for synop stations.

INPUT: LIST OF NECESSARY PARAMETERS:

- varMETnum – meteorological variable code in ODB (for example, 39 is for 2 metre air temperature in ODB)

- varMETstr – corresponding meteorological variable code in FDB (for example, 2t is in FDB)

- lon_val_min – western boundary of selected geographical domain (for example, 20)

- lon_val_max – eastern boundary of selected geographical domain (for example, 25)

- lat_val_min – southern boundary of selected geographical domain (for example, 60)

- lat_val_max – northern boundary of selected geographical domain (for example, 65)

This script calls subsequently to

- gsv_mod_data.sh – to pre-process necessary extracted modeled data with gsv_interface

- synop_obs.sh – to extract observation data (from ODB) at geographical locations (latitude, longitude) for each synop station

- synop_mod.sh – to extract and interpolate modeled data at exact locations of synop stations, and to add interpolated modelled data to ODB

- graph_mod_obs.py – to plot differences between observation and modelled data (“for self-control”; can be ignored)

- synop_stats.sh – to calculate relevant statistics and produce a series of plot (see Figures 1abc)

PROCESSING STEPS:

- gsv_mod_data.sh - bash-script to run pre-processing (using cdo operators) of modelled data extracted with gsv interface over global domain for selected meteorological variables (measured at synop station) into hourly time-slices and saving (temporary) to separate hourly nc-files.

- synop_obs.sh - bash-script to run reading and extraction (using sql with odb_api software) of selected meteorological variables at hourly time-slices for observations from ground-based synop stations at fixed at the surface geographical (latitude, longitude) locations over selected geographical domain (or over the globe) and saving (temporary) to dat-file. Note, that list of available  meteorological variables in open multi-year data ODB includes: 91 - total amount of clouds; 108 - sea level pressure; 80 - 1-hour precipitation; 58 - relative humidity; 999 - 10-minute precipitation intensity; 71 - snow depth; 39 - 2m temperature; 40 - dew point temperature; 62 - visibility; 111 - wind direction; 261 - maximum wind gust in last 10 minutes; 221 -surface wind speed.

- synop_mod.sh - bash-script to run reading, extraction and interpolation (using cdo operators) of modelled data for selected meteorological variables at the same hourly time-slices to geographical (latitude, longitude) locations of synop stations, and saving (temporary) to dat-file, and adding (using import with odb_api) such data to ODB. Interpolation is preceded by constructing unstructured grid based on locations of synop stations and calculating weights for such grid.

- graph_mod_obs.py - python-script to run an internal self-control in calculating differences between observed and modelled data for locations of synop stations.

- synop_stats.sh – to calculate relevant statistics and produce a series of plots (see section 3.2 for details).

PATH TO ALL FILES ABOVE IS: obsall/SYNOP

## Monitoring for SYNOP Part

synop_stats.sh - bash-script to run (additional bash, fortran, python scripts) and calculate necessary relevant statistics and produce a series of relevant plots for synop stations.

This script calls subsequently to

- produce_rank_histograms_all_stations.sh – to access necessary data, calculate quantile rank histograms and their summary statistics for each synop station for 00, 06, 12, and 18 UTCs (see section 3.2.1).

- produce_standard_plots_all_stations.sh – to access required data and to call/plot quantiles rank histogram for each synop station (see section 3.2.2).

- summary_rank_histograms_all_stations.sh – to accessing required data, to calculate rank histogram summary statistics over all synop stations, to call/plot p-values over geographical domain for all stations and rank histogram summary statistics based on all synop stations (see section 3.2.3).

### 3.2.1. Rank histograms for all synop stations for 00, 06, 12, and 18 UTCs

(with produce_rank_histograms_all_stations.sh, using rank_histograms_one_station.f95 and sql/import with odb_api)

FILE (BASH-SCRIPT): obsall/SYNOP/STATS/produce_rank_histograms_all_stations.sh is used for calculation of quantile space rank histograms and their summary statistics (Mean Square Deviation, MSD and p-value) for all synop stations for 00, 06, 12, and 18 UTCs.

PROCESSING STEPS:

- Passing list of necessary parameters from the synop_stats.sh script

- Accessing raw data from model simulations, quantiles and bootstrap MSD statistics

- Accessing file with list of synop stations and their geographical coordinates

- Compiling fortran program rank_histograms_one_station.f95 to calculate rank histograms

- Initiating loop over all synop stations

- Selecting the simulation data for synop station

- Selecting the quantiles for synop station

- Selecting the MSD for synop station

- Producing the rank histogram for synop station for 00, 06, 12, and 18 UTCs

- Converting the rank histogram file to ODB format

- Combining the rank histogram data to all synop stations in a single ODB file

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – meteorological variable code in ODB (for example, 39 is for 2 metre air temperature in ODB)

- year1 – first year (for example, 2020)

- year2 – last year (for example, 2020)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

Note: ${period} is calculated based on year1, year2, month1, month2

LIST OF INPUT DATA REQUIRED:

- file incl. list of synop stations (it is assumed that the list of such stations has been pre-produced)

obsall/SYNOP/STATS/list_of_stations_${variable}.txt

- path to location of raw simulation data at coordinates of synop stations

sim_dir=/scratch/project_465000454/ama/open_data_ODB

- path to location of file incl. quantiles as a function of time of year

quant_dir=/scratch/project_465000454/ama/STATDATASYNOP/quantiles

- path to location of file incl. rank histogram bootstrap MSD values

bootstrap_dir=/scratch/project_465000454/ama/STATDATASYNOP/bootstrap_statistics/${variable}_${period}

OUTPUT FOR RESULTS:

- path to store calculated output for each synop station in separate files

outdir= obsall/SYNOP/STATS/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

outfile=${outdir}/rank_histograms_${variable}_${station}_${year1}${month1}-${year2}${month2}

Note: ${station} is read from file list_of_stations_${variable}.txt

FILE (FORTRAN PROGRAM): obsall/SYNOP/STATS/fortran-programs/rank_histograms_one_station.f95 is called and compiled from produce_rank_histograms_all_stations.sh and calculates for each synop station the frequencies of quantiles having output incl. station code/ID, longitude, latitude, UTC hour, number of realizations for this hour, MSD for the quantile frequencies, p-value of MSD (=fraction of bootstrap realizations with larger or equal MSD), and quantile frequencies (on scale 0-1) from 0-1% ... 99-100%.

### 3.2.2. Standard plots for each synop station

(with produce_standard_plots_all_stations.sh using plot_quantiles_rankhist_synop.py and sql with odb_api; see an example of such plot in Figure 1a)

FILE (BASH-SCRIPT): obsall/SYNOP/STATS/produce_standard_plots_all_stations.sh is used for accessing required data and calling for plotting quantiles rank histogram for each individual synop station.

PROCESSING STEPS:

- Passing list of necessary parameters from the synop_stats.sh script

- Accessing raw data from model simulations, observations, quantiles, and calculated rank histogram data

- Accessing file with list of synop stations and their geographical coordinates

- Initiating loop over all synop stations

- Selecting the simulation data for synop station

- Selecting the rank histogram data for synop station

- Selecting the quantiles for synop station

- Calling python script plot_quantiles_rankhist_synop.py to plot quantiles rank histogram for synop station

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – meteorological variable code in ODB (for example, 39 is for 2 metre air temperature in ODB)

- year1 – first year (for example, 2020)

- year2 – last year (for example, 2020)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

LIST OF INPUT DATA REQUIRED:

- file incl. list of synop stations (it is assumed that the list of such stations has been pre-produced)

obsall/SYNOP/STATS/list_of_stations_${variable}.txt

- path to location of raw simulation data at coordinates of synop stations

sim_dir=/scratch/project_465000454/ama/open_data_ODB

- path to location of file incl. quantiles as a function of time of year

quant_dir=/scratch/project_465000454/ama/STATDATASYNOP/quantiles

- path to location of file incl. computed rank histogram data

rh_dir= obsall/SYNOP/STATS/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

OUTPUT FOR RESULTS (PLOTS):

- path to store calculated output for each synop station in separate files

figure_dir=obsall/SYNOP/STATS/figures/standard_plots_${variable}_${year1}${month1}-${year2}${month2}

fig_name=${figure_dir}/standard_plot_${variable}_${station}_${year1}${month1}-${year2}${month2}_python.png

Note: ${station} is read from file list_of_stations_${variable}.txt

FILE (PYTHON PROGRAM): obsall/SYNOP/STATS/python/plot_quantiles_rankhist_synop.py produces individual plot (see, for example, Figure 1a “Quantiles/ time series data (00, 06, 12, and 18 UTCs) and rank histogram (combination of 00, 06, 12, and 18 UTCs) synop stationID”) for each synop station. Python program uses passed list of parameters:  quantiles_data, rank_data, time_series_data, variable, station_id, lon, lat, year_beg, year_end, month_beg, month_end, figname from the produce_standard_plots_all_stations.sh bash-script.

Figure 1a: Plot for 2 metre air temperature (T2m) for period of 1 Jan - 31 Dec 2020: Quantiles/ time series data (00, 06, 12, and 18 UTCs) and rank histogram (combination of 00, 06, 12, and 18 UTCs) for synop station 126736.

### 3.2.3. Summary rank histograms for all synop stations

(with summary_rank_histograms_all_stations.sh using rank_histogram_summary_statistics.f95, plot_p_values_map_synop.py, plot_rankhist_sum_all_stations_synop.py and sql with odb_api; see examples of such plots in Figure 1b and Figure 1c).

FILE (BASH-SCRIPT): obsall/SYNOP/STATS/summary_rank_histograms_all_stations.sh is used for accessing required data, calculating rank histogram summary statistics over all synop stations, calling for plotting p-values over geographical domain for all stations, and calling for plotting rank histogram summary statistics based on all synop stations.

PROCESSING STEPS:

- Passing list of necessary parameters from the synop_stats.sh script

- Accessing rank histogram data

- Compiling fortran program rank_histogram_summary_statistics.f95 to calculate rank histogram summary statistics

- Calling python script plot_p_values_map_synop.py to plot p-values for all synop stations over geographical domain

- Calling python script plot_rankhist_sum_all_stations_synop.py to plot rank histogram summary statistics based on all synop stations

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – meteorological variable code in ODB (for example, 39 is for 2 metre air temperature in ODB)

- year1 – first year (for example, 2020)

- year2 – last year (for example, 2020)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

LIST OF INPUT DATA REQUIRED:

- path to location of file incl. computed rank histogram data

rh_dir= obsall/SYNOP/STATS/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

OUTPUT FOR RESULTS (PLOTS):

- path to store plotted output (based on all synop stations) in separate files

figure_dir= obsall/SYNOP/figures

figure_dir/p-value_map_${variable}_${year1}${month1}-${year2}${month2}.png

figure_dir/rank-hist-sumstats_${variable}_${year1}${month1}-${year2}${month2}.png

FILE (PYTHON PROGRAM): obsall/SYNOP/STATS/python/plot_p_values_map_synop.py produces a single plot (see, for example, Figure 1b “P-values for 2 metre air temperature quantiles”). Python program uses passed list of parameters:  p_values_file, p_values from the summary_rank_histograms_all_stations.sh bash-script.

FILE (PYTHON PROGRAM): obsall/SYNOP/STATS/pyhon/plot_rankhist_sum_all_stations_synop.py produces a single plot (see, for example, Figure 1c “Rank histogram summary statistics for 2 metre air temperature: normalized p-value frequencies and normalized quantile frequencies”). Python program uses passed list of parameters: p_freq, q_freq, number_of_stations, p01, p1, p5, max_freq_p, and max_freq_q from the summary_rank_histograms_all_stations.sh bash-script.

Figure 1b: Plot for 2 metre air temperature (2t) for period of 1 Jan - 31 Dec 2020: P-values for 2t quantiles.

Figure 1c: Plot for 2 metre air temperature (2t) for period of 1 Jan - 31 Dec 2020: Rank histogram summary statistics for 2t: normalized p-value frequencies and normalized quantile frequencies.

## Part TEMP (radiosounding observations)

main_radsound.sh - main bash-script to run Apps for observational data for radiosounding stations.

INPUT: LIST OF NECESSARY PARAMETERS:

- varMETnum – meteorological variable code in ODB (for example, 39 is for 2 metre air temperature in ODB)

- varMETstr – corresponding meteorological variable code in FDB (for example, 2t is in FDB)

- lon_val_min – western boundary of selected geographical domain (for example, 20)

- lon_val_max – eastern boundary of selected geographical domain (for example, 25)

- lat_val_min – southern boundary of selected geographical domain (for example, 60)

- lat_val_max – northern boundary of selected geographical domain (for example, 65)

This script calls subsequently to

- gsv_radsound_mod_data.sh – to pre-process necessary extracted modeled data with gsv_interface

- radsound_obs.sh – to extract observation data (from ODB) at geographical locations (latitude, longitude, pressure level) for each radiosounding station

- radsound_mod.sh – to extract and interpolate modeled data at exact locations of radiosounding stations, and to add interpolated modelled data to ODB

- radsound_stats.sh – to calculate relevant statistics and produce a series of plot (see Figures 2abc)

PROCESSING STEPS:

- gsv_radsound_mod_data.sh - bash-script to run pre-processing (using cdo operators) of modelled data extracted with gsv interface over global domain for air temperature at 2 metre (T2) and at 850 hPa pressure level (T850) into time-slices at 00 and 12 UTCs, calculating difference (T2-T850) between these two temperatures, and saving (temporary) to separate nc-files at 00 and 12 UTC time-slices.

- radsound_obs.sh - bash-script to run reading and extraction (using sql with odb_api software) of above-mentioned air temperatures at 12 hourly time-slices for radiosounding observations at surface and at altitude at geographical (latitude, longitude, pressure level) locations of radiosounding stations over selected geographical domain (or over the globe), calculating difference between two temperatures, and saving (temporary) to dat-file.

- radsound_mod.sh - bash-script to run reading, extraction and interpolation (cdo/ later using polytope) of modelled data for air temperature at surface and at altitude at the same time-slices to geographical (latitude, longitude, pressure level) locations of radiosounding stations, calculating difference between two temperatures, and saving (temporary) to dat-file, and adding (using import with odb_api) such data to ODB.

- radsound_stats.sh – to calculate relevant statistics and produce a series of plots (see section 3.4 for details).

PATH TO ALL FILES ABOVE IS: obsall/RADSOUND

## Monitoring for TEMP Part

radsound_stats.sh - bash-script to run (additional bash, fortran, python scripts) and calculate necessary relevant statistics and produce a series of relevant plots for radiosounding stations.

This script calls subsequently to

- produce_rank_histograms_all_stations.sh – to access necessary data, calculate quantile rank histograms and their summary statistics for each radiosounding station for 00 and 12 UTCs (see section 3.4.1).

- produce_standard_plots_all_stations.sh – to access required data and to call/plot quantiles rank histogram for each radiosounding station (see section 3.4.2).

- summary_rank_histograms_all_stations.sh – to accessing required data, to calculate rank histogram summary statistics over all radiosounding stations, to call/plot p-values over geographical domain for all stations and rank histogram summary statistics based on all radiosounding stations (see section 3.4.3).

### 3.4.1. Rank histograms for all radiosounding stations for 00 and 12 UTCs

(with produce_rank_histograms_all_stations.sh, using rank_histograms_one_station.f95 and sql/import with odb_api)

FILE (BASH-SCRIPT): obsall/RADSOUND/STATRS/produce_rank_histograms_all_stations.sh is used for calculation of quantile space rank histograms and their summary statistics (Mean Square Deviation, MSD and p-value) for all radiosounding stations for 00 and 12 UTCs.

PROCESSING STEPS:

- Passing list of necessary parameters from the synop_stats.sh script

- Accessing raw data from model simulations, quantiles and bootstrap MSD statistics

- Accessing file with list of radiosounding stations and their geographical coordinates

- Compiling fortran program rank_histograms_one_station.f95 to calculate rank histograms

- Initiating loop over all radiosounding stations

- Selecting the simulation data for radiosounding station

- Selecting the quantiles for radiosounding station

- Selecting the MSD for radiosounding station

- Producing the rank histogram for radiosounding station for 00 and 12 UTCs

- Converting the rank histogram file to ODB format

- Combining the rank histogram data to all radiosounding stations in a single ODB file

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – “T2-T850” (difference between the air temperatures at 2 metre and 850 hPa level)

Note: T2-T850 is only one variable used for radiosounding data

- year1 – first year (for example, 2010)

- year2 – last year (for example, 2010)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

Note: ${period} is calculated based on year1, year2, month1, month2

LIST OF INPUT DATA REQUIRED:

- file incl. list of radiosounding stations (it is assumed that the list of such stations has been pre-produced & includes stations with, at least, 1000 observation data points)

obsall/RADSOUND/STATRS/list_of_valid_radsound_stations.txt

- path to location of raw simulation data at coordinates of radiosounding stations

sim_dir= scratch/project_465000454/ama/STATDATARADSOUND/RHARM_vertical_differences_odb

- path to location of file incl. quantiles as a function of time of year

quant_dir=/scratch/project_465000454/ama/STATDATARADSOUND/quantiles

- path to location of file incl. rank histogram bootstrap MSD values

bootstrap_dir=/scratch/project_465000454/ama/STATDATARADSOUND/bootstrap_statistics/${variable}_${period}

OUTPUT FOR RESULTS:

- path to store calculated output for each radiosounding station in separate files

outdir= obsall/RADSOUND/STATRS/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

outfile=${outdir}/rank_histograms_${variable}_${station}_${year1}${month1}-${year2}${month2}

Note: ${station} is read from file list_of_valid_radsound_stations.txt

FILE (FORTRAN PROGRAM): obsall/RADSOUND/STATRS/fortran-programs/rank_histograms_one_ station.f95 is called and compiled from produce_rank_histograms_all_stations.sh and calculates for each synop station the frequencies of quantiles having output incl. station code/ID, longitude, latitude, UTC hour, number of realizations for this hour, MSD for the quantile frequencies, p-value of MSD (=fraction of bootstrap realizations with larger or equal MSD), and quantile frequencies (on scale 0-1) from 0-1% ... 99-100%.

### 3.4.2. Standard plots for each radiosounding station

(with produce_standard_plots_all_stations.sh using plot_quantiles_rankhist_temp.py and sql with odb_api; see an example of such plot in Figure 2a)

FILE (BASH-SCRIPT): obsall/RADSOUND/STATRS/produce_standard_plots_all_stations.sh is used for accessing required data and calling for plotting quantiles rank histogram for each individual radiosounding station.

PROCESSING STEPS:

- Passing list of necessary parameters from the radsound_stats.sh script

- Accessing raw data from model simulations, observations, quantiles, and calculated rank histogram data

- Accessing file with list of radiosounding stations and their geographical coordinates

- Initiating loop over all radiosounding stations

- Selecting the simulation data for radiosounding station

- Selecting the rank histogram data for radiosounding station

- Selecting the quantiles for radiosounding station

- Calling python script plot_quantiles_rankhist_temp.py to plot quantiles rank histogram for radiosounding station

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – “T2-T850” (difference between the air temperatures at 2 metre and 850 hPa level)

Note: T2-T850 is only one variable used for radiosounding data

- year1 – first year (for example, 2010)

- year2 – last year (for example, 2010)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

LIST OF INPUT DATA REQUIRED:

- file incl. list of radiosounding stations (it is assumed that the list of such stations has been pre-produced & includes stations with, at least, 1000 observation data points)

obsall/RADSOUND/STATRS/list_of_valid_radsound_stations.txt

- path to location of raw simulation data at coordinates of radiosounding stations

sim_dir= /scratch/project_465000454/ama/STATDATARADSOUND/RHARM_vertical_differences_odb

- path to location of file incl. quantiles as a function of time of year

quant_dir=/scratch/project_465000454/ama/STATDATARADSOUND

- path to location of file incl. computed rank histogram data

rh_dir= obsall/RADSOUND/STATRS/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

OUTPUT FOR RESULTS (PLOTS):

- path to store calculated output for each radiosounding station in separate files

figure_dir=obsall/RADSOUND/STATRS/figures/standard_plots_${variable}_${year1}${month1}-${year2}${month2}

fig_name=${figure_dir}/standard_plot_${variable}_${station}_${year1}${month1}-${year2}${month2}.png

Note: ${station} is read from file list_of_valid_radsound_stations.txt

FILE (PYTHON PROGRAM): obsall/RADSOUND/STATRS/python/plot_quantiles_rankhist_temp.py produces individual plot (see, for example, Figure 2a “Quantiles/ time series data and rank histogram (00 and 12 UTCs) radiosounding stationID”) for each radiosounding station. Python program uses passed list of parameters: quantiles_data, rank_data, time_series_data, station_id, lon, lat, year_beg, year_end, month_beg, month_end, fig_name from the produce_standard_plots_all_stations.sh bash-script.

Figure 2a: Plots for difference in air temperatures at 2 metre and at 850 hPa pressure level (T2-T850) for period of 1 Jan – 31 Dec 2010: Quantiles/ time series data and rank histogram (00 and 12 UTCs) for radiosounding station 51431.

### 3.4.3. Summary rank histograms for all radiosounding stations

(with summary_rank_histograms_all_stations.sh using rank_histogram_summary_statistics.f95, plot_p_values_map_temp.py, plot_rankhist_sum_all_stations_temp.py and sql with odb_api; see examples of such plots in Figure 1b and Figure 1c).

FILE (BASH-SCRIPT): obsall/RADSOUND/STATRS/summary_rank_histograms_all_stations.sh is used for accessing required data, calculating rank histogram summary statistics over all radiosounding stations, calling for plotting p-values over geographical domain for all stations, and calling for plotting rank histogram summary statistics based on all radiosounding stations.

PROCESSING STEPS:

- Passing list of necessary parameters from the radsound_stats.sh script

- Accessing rank histogram data

- Compiling fortran program rank_histogram_summary_statistics.f95 to calculate rank histogram summary statistics for 00 and 12 UTCs

- Calling python script plot_p_values_map_temp.py to plot p-values for all radiosounding stations over geographical domain for 00 and 12 UCTs

- Calling python script plot_rankhist_sum_all_stations_temp.py to plot rank histogram summary statistics based on all radiosounding stations or 00 and 12 UTCS

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – “T2-T850” (difference between the air temperatures at 2 metre and 850 hPa level)

Note: T2-T850 is only one variable used for radiosounding data

- year1 – first year (for example, 2010)

- year2 – last year (for example, 2010)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

LIST OF INPUT DATA REQUIRED:

- path to location of file incl. computed rank histogram data

rh_dir= obsall/RADSOUND/STATRS/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

OUTPUT FOR RESULTS (PLOTS):

- path to store plotted output (based on all radiosounding stations) in separate files

figure_dir= obsall/RADSOUND/STATRS/figures

figure_dir/p-values_rank-histograms_${variable}_${year1}${month1}-${year2}${month2}.png

figure_dir/summary_plot_rank-histograms_${variable}_${year1}${month1}-${year2}${month2}.png

FILE (PYTHON PROGRAM): obsall/RADSOUND/STATRS/python/plot_p_values_map_temp.py produces a single plot (see, for example, Figure 2b “P-values for T2-T850 quantiles”). Python program uses passed list of parameters:  p_values_file, p_values_00, p_values_12 from the summary_rank_histograms_all_stations.sh bash-script.

FILE (PYTHON PROGRAM): obsall/RADSOUND/STATRS/python/plot_rankhist_sum_all_stations_temp.py produces a single plot (see, for example, Figure 2c “Rank histogram summary statistics for T2-T850 for 00 and 12 UTCs: normalized p-value frequencies and normalized quantile frequencies”). Python program uses passed list of parameters: p_freq_00, p_freq_12, q_freq_00, q_freq_12, number_of_stations, p01_00, p1_00, p5_00, p01_12, p1_12, p5_12, p_ylim, q_ylim, figname from the summary_rank_histograms_all_stations.sh bash-script.

Figure 2b: Plots for difference in air temperatures at 2 metre and at 850 hPa pressure level (T2-T850) for period of 1 Jan - 31 Dec 2010: P-values for T2-T850 quantiles.

Figure 2c: Plots for difference in air temperatures at 2 metre and at 850 hPa pressure level (T2-T850) for period of 1 Jan - 31 Dec 2010: Rank histogram summary statistics for 00 and 12 UTCs: normalized p-value frequencies and normalized quantile frequencies.

## Part AMSUA-A (satellite observations)

main_amsua.sh - main bash-script to run Apps for observational data for satellites.

INPUT: LIST OF NECESSARY PARAMETERS:

- varMETname – meteorological variable name (btmp – brightness temperature)

- varMETnum – meteorological variable code (194 for btmp)

- channel – Satellite AMSU-A channel number (5)

This script calls subsequently to

- set_env.sh – bash-script which defines variables and functions, and required modules to be loaded and used in the workflow.

- gsv_mod_data.sh – to pre-process necessary extracted modeled data from sfc-, pl-, and o2d-levels with gsv_interface over area, interpolate from pl- to model-levels

- amsua_obs.sh – to prepare metadata for computation of AMSU-A model counterparts for each area

- amsua_mod.sh – to run the Radiance Simulator and to produce a model counterpart, and add interpolated modelled data to ODB

- amsua_stats.sh – to calculate relevant statistics and produce a series of plot (see Figures 3abc)

PROCESSING STEPS:

- gsv_mod_data.sh – bash-script to run pre-processing (using cdo operators; in later versions - polytope) of 2D on surface (sfc) and 3D modelled data on pressure levels (pl) extracted with gsv interface over global domain. The Radiance Simulator (using convert_pl2ml.py) interpolates modelled data from pl-levels to “model”-levels (note, workflow will initially be demonstrated on a single selected location i.e., around a point of 70.0N, 45.0E in the Barents Sea; and later geographically expanded using polytope) and preprocessed data saved in (temporary) hourly GRIB-files. Required 3D variables are: t - temperature (130), q - specific humidity (133), clwc - specific liquid water content (246); and 2D variables are: skt - skin temperature (235), ci - sea ice fraction (31), lsm - land-sea mask (172), sp - surface pressure (134), 10u and 10v - U and V components of wind speed at 10 m (165 and 166), 2t - air temperature at 2 m (167), 2d - dewpoint temperature at 2 m (168), and z - geopotential (for orography) (129).

- amsua_obs.sh - bash-script to prepare a metadata file for computation of AMSU-A model counterparts from preprocessed hourly GSV data. It runs depending on the type of climate simulation: (i) reality following data assimilation-like, and (ii) free running climate simulation. For (i) the one-to-one model counterparts can be produced, so actual metadata from AMSU-A observations in the vicinity of the selected location are extracted from ODB. This is done with filter_data.py. For (ii) the model output can be compared to observations only in climatological terms, so metadata is written only for the centre point of the selected area.

- amsua_mod.sh - bash-script to run the Radiance Simulator tool with pre-described control configuration parameters and to produce a model counterpart (file in netcdf-format), which (using convert_netcdf2txt.py) is converted to (temporary) dat-file and added (using import with odb_api) to ODB.

- amsua_stats.sh – to calculate relevant statistics and produce a series of plots (see section 3.6 for details).

PATH TO ALL FILES ABOVE IS: obsall/SATELLITE/STATST

## Monitoring for AMSU-A Part

amsua_stats.sh - bash-script to run (additional bash, fortran, python scripts) and calculate necessary relevant statistics and produce a series of relevant plots for selected areas, for example, are “a00” (with a size of about 100x100 km and assigned to exact geographical locations, i.e., to the center point of the area)

This script calls subsequently to

- produce_rank_histograms_all_locations.sh – to access necessary data, calculate quantile rank histograms and their summary statistics for all areas from satellite observations for daily mean values of brightness temperature (see section 3.6.1).

- produce_standard_plots_all_locations.sh – to access required data and to call/plot quantiles rank histogram for each area (see section 3.6.2).

- summary_rank_histograms_all_locations.sh – to accessing required data, to calculate rank histogram summary statistics over all areas, to call/plot p-values over geographical domain for all areas and rank histogram summary statistics based on all areas (see section 3.6.3).

### 3.6.1. Rank histograms for all areas for daily mean

(with produce_rank_histograms_all_locations.sh, using rank_histograms_one_location.f95 and daymean.py and sql/ import with odb_api),

FILE (BASH-SCRIPT): obsall/SATELLITE/STATST/produce_rank_histograms_all_locations.sh is used for calculation of quantile space rank histograms and their summary statistics (Mean Square Deviation, MSD and p-value) for areas for the daily mean brightness temperature (btmp).

PROCESSING STEPS:

- Passing list of necessary parameters from the amsua_stats.sh script

- Accessing raw data from model simulations, quantiles and bootstrap MSD statistics

- Accessing file with list of areas and their geographical coordinates (i.e., centers of areas)

- Compiling fortran program rank_histograms_one_location.f95 to calculate rank histograms

- Initiating loop over all areas

- Selecting the simulation data for area

- Selecting the quantiles for area

- Selecting the MSD for area

- Calling python script daymean.py to produce daily mean value of brightness temperature

- Producing the rank histogram for area

- Converting the rank histogram file to ODB format

- Combining the rank histogram data to all areas in a single ODB file

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – Channel number of AMSU-A satellite observations

Note: currently only the channel number 5 is used

Note: currently only the brightness temperature, btmp (194), is evaluated

- year1 – first year (for example, 2016)

- year2 – last year (for example, 2016)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

LIST OF INPUT DATA REQUIRED:

- file incl. list of locations for areas (it is assumed that the list of such locations has been pre-produced)

Note:  currently there is only 1 area called "a00" (with a size of about 100x100km over the Barents Sea) and which is assigned to exact geographical location (45 deg E & 70 deg N) i.e., in the center point of the area “a00”

obsall/SATELLITE/STATST/list_of_all_amsua_locations.txt

- path to location of raw simulation data for areas

sim_dir=/scratch/project_465000454/ama/AMSUA_demo_ODB/test_odb_files

- path to location of file incl. quantiles as a function of time of year

quant_dir=/scratch/project_465000454/ama/STATDATASATELLITE/quantiles

- path to location of file incl. rank histogram bootstrap MSD values

bootstrap_dir=/scratch/project_465000454/ama/STATDATASATELLITE/bootstrap_statistics

OUTPUT FOR RESULTS:

- path to store calculated output for each area in separate files

outdir= obsall/SATELLITE/STATST/rank_histograms/${variable}_${year1}${month1}-${year2}${month2} outfile=${outdir}/rank_histograms_${variable}_${station}_${year1}${month1}-${year2}${month2}

FILE (FORTRAN PROGRAM): obsall/SATELLITE/STATST/rank_histograms_one_location.f95 is called and compiled from produce_rank_histograms_all_locations.sh and calculates for each area the frequencies of quantiles having output incl. area code/ID, longitude, latitude, hour, MSD for the quantile frequencies, p-value of MSD (=fraction of bootstrap realizations with larger or equal MSD), and quantile frequencies (on scale 0-1) from 0-1% ... 99-100%.

FILE (PYTHON PROGRAM): obsall/SATELLITE/STATST/python/daymean.py calculates daily mean brightness temperature, btmp. Python program uses passed list of parameters:  hourly_data_file, daily_data_file from the produce_rank_histograms_all_locations.sh bash-script.

### 3.6.2. Standard plots for each area

(with produce_standard_plots_all_locations.sh using plot_quantiles_rankhist_amsua.py and sql with odb_api; see an example of such plot in Figure 3a)

FILE (BASH-SCRIPT): obsall/SATELLITE/STATST/produce_standard_plots_all_stations.sh is used for accessing required data and calling for plotting quantiles rank histogram for each individual area.

PROCESSING STEPS:

- Passing list of necessary parameters from the amsua_stats.sh script

- Accessing raw data from model simulations, observations, quantiles, and calculated rank histogram data

- Accessing file with list of areas and geographical coordinates of centres of these areas

- Initiating loop over all areas

- Selecting the simulation data for areas

- Selecting the rank histogram data for areas

- Selecting the quantiles for areas

- Calling python script plot_quantiles_rankhist_amsua.py to plot quantiles rank histogram for areas

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – Channel number of AMSU-A satellite observations

Note: currently only the channel number 5 is used

Note: currently only the brightness temperature, btmp (194), is evaluated

- year1 – first year (for example, 2016)

- year2 – last year (for example, 2016)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

LIST OF INPUT DATA REQUIRED:

- file incl. list of locations for areas (it is assumed that the list of such locations has been pre-produced)

Note:  currently there is only 1 area called "a00" (with a size of about 100x100km over the Barents Sea) and which is assigned to exact geographical location (45 deg E & 70 deg N) i.e., in the center point of the area “a00”

obsall/SATELLITE/STATST/list_of_all_amsua_locations.txt

- path to location of raw simulation data for areas

sim_dir=/scratch/project_465000454/ama/AMSUA_demo_ODB/test_odb_files

- path to location of file incl. quantiles as a function of time of year

quant_dir=/scratch/project_465000454/ama/STATDATASATELLITE/quantiles

- path to location of file incl. computed rank histogram data

rh_dir= obsall/SATELLITE/STATST/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

OUTPUT FOR RESULTS (PLOTS):

- path to store calculated output for each area in separate files

figure_dir=obsall/SATELLITE/STATST/figures/standard_plots_${variable}_${year1}${month1}-${year2}${month2}

fig_name=${figure_dir}/standard_plot_${variable}_${station}_${year1}${month1}-${year2}${month2}.png

Note: ${station} is read from file list_of_all_amsua_locations.txt

FILE (PYTHON PROGRAM): obsall/SATELLITE/STATST/python/plot_quantiles_rankhist_amsua.py produces individual plot (see, for example, Figure 3a “Quantiles/ time series data and rank histogram for daily mean brightness temperature for areaID”) for each area. Python program uses passed list of parameters: quantiles_data, rank_data, time_series_data, fig_title, fig_name, year_beg, year_end, month_beg, month_end, station_id from the produce_standard_plots_all_locations.sh bash-script.

Figure 3a: Plots for brightness temperature /AMSU-A Channel 5/ for selected geographical area “a00” for period of 1 Jan - 31 Dec 2016: Quantiles/ time series data and rank histogram for daily mean brightness temperature for area “a00”.

### 3.6.3. Summary rank histograms for all areas

(with summary_rank_histograms_all_locations.sh using rank_histogram_sumstats_locations.f95, plot_p_values_map_amsua.py, plot_rankhist_sum_all_areas_amsua.py and sql with odb_api; see examples of such plots in Figure 3b and Figure 3c).

FILE (BASH-SCRIPT): obsall/SATELLITE/STATST/summary_rank_histograms_all_locations.sh is used for accessing required data, calculating rank histogram summary statistics over all areas, calling for plotting p-values over geographical domain for all areas, and calling for plotting rank histogram summary statistics based on all areas.

PROCESSING STEPS:

- Passing list of necessary parameters from the amsua_stats.sh script

- Accessing rank histogram data

- Compiling fortran program rank_histogram_sumstats_locations.f95 to calculate rank histogram summary statistics for daily mean brightness temperature for areas

- Calling python script plot_p_values_map_amsua.py to plot p-values for daily man brightness temperature for all areas over geographical domain

- Calling python script plot_rankhist_sum_all_areas_amsua.py to plot rank histogram summary statistics for daily mean brightness temperature based on all areas

INPUT: LIST OF NECESSARY PASSED PARAMETERS:

- variable – Channel number of AMSU-A satellite observations

Note: currently only the channel number 5 is used

Note: currently only the brightness temperature, btmp (194), is evaluated

- year1 – first year (for example, 2016)

- year2 – last year (for example, 2016)

- month1 – first month (for example, 01)

- month2 – last month (for example, 12)

LIST OF INPUT DATA REQUIRED:

- path to location of file incl. computed rank histogram data

rh_dir= obsall/SATELLITE/STATST/rank_histograms/${variable}_${year1}${month1}-${year2}${month2}

OUTPUT FOR RESULTS (PLOTS):

- path to store plotted output (based on all areas) in separate files

figure_dir= obsall/SATELLITE/STATST/figures

figure_dir/p-values_map_amsua_${variable}_${year1}${month1}-${year2}${month2}.png

figure_dir/rankhist_sumstats_amsua_${variable}_${year1}${month1}-${year2}${month2}.png

FILE (PYTHON PROGRAM): obsall/SATELLITE/STATST/python/plot_p_values_map_amsua.py produces a single plot (see, for example, Figure 3b “P-values for brightness temperature quantiles for areas”). Python program uses passed list of parameters:  p_values_file, p_values from the summary_rank_histograms_all_locations.sh bash-script.

FILE (PYTHON PROGRAM): obsall/SATELLITE/STATST/python/plot_rankhist_sum_all_areas_amsua.py produces a single plot (see, for example, Figure 3c “Rank histogram summary statistics for brightness temperature for areas: normalized p-value frequencies and normalized quantile frequencies”). Python program uses passed list of parameters: p_freq, q_freq, number_of_stations, p01, p1, p5, max_freq_p, max_freq_q from the summary_rank_histograms_all_locations.sh bash-script.

Figure 3b: Plots for brightness temperature /AMSU-A Channel 5/ for selected geographical area “a00” for period of 1 Jan - 31 Dec 2016: P-values for brightness temperature quantiles for areas. Note: on Figure 3b the plotted dot as a colored circle represents the central point (70 N, 45 E) of the area "a00", which is situated within boundaries of [69.5, 70.5] N and [43.5, 46.5] E.

Figure 3c: Plots for brightness temperature /AMSU-A Channel 5/ for selected geographical area “a00” for period of 1 Jan - 31 Dec 2016: Rank histogram summary statistics for brightness temperature for areas: normalized p-value frequencies and normalized quantile frequencies.

# CHAPTER 4: CONTRIBUTING TO OBSALL

Contributions to the OBSALL Apps are always welcome! These inlude reporting on encountered bugs and issues, suggesting new features and enhancements, contributing to code and documentation. The sections below outline process for contributing to OBSALL.

## Reporting bugs

In case any bugs or issues are encountered while using OBSALL; please, follow steps below to report:

1. Navigate to the OBSALL GitLab repository (https://earth.bsc.es/gitlab/digital-twins/de_340/obsall).

2. Click on the “Issues” tab.

3. Click on the “New issue” button.

4. Choose to provide a title, type of issue, and detailed description of the issue, including steps to reproduce, expected behavior, and any relevant error messages & click on the “Create issue” button.

## Suggesting features

Any idea for new feature or enhancement in OBSALL is welcome; please, follow steps below to suggest:

1. Navigate to the OBSALL GitLab repository (https://earth.bsc.es/gitlab/digital-twins/de_340/obsall).

2. Click on the “Issues” tab.

3. Click on the “New issue” button.

4. Choose to provide a title, type of issue, and detailed description of the proposed feature, including use cases, benefits, and potential challenges & click on the “Create issue” button.

## Contributing to code

Contributions to the OBSALL code (SYNOP, TEMP, AMSU-A Parts and their monitoring/ statistics) are welcome; please, follow steps below to contribute:

1. Ask the OBSALL development team to grant you access to the OBSALL repository at GitLab.

2. Navigate to the OBSALL GitLab repository (https://earth.bsc.es/gitlab/digital-twins/de_340/obsall) & create an issue (following steps from 4.2) to discuss proposed changes/ features.

3. Clone the OBSALL repository to your local machine (e.g., git clone https://earth.bsc.es/gitlab/digital-twins/de_340/obsall.git & cd obsall)

4. Create a new branch for feature or bugfix (e.g., git checkout -b my-branch).

5. Make and commit your changes (git status & git add file-name-with-changes-made & git commit -m "write-in-short-about-changes-made”)

6. When ready, push your branch to the OBSALL repository (git push origin my-branch)

7. Create a pull request in the OBSALL repository, describing your changes and referencing any related issues.

8. Once your pull request is approved, it will be merged into the main branch by the OBSALL development team.

Warning: Please, do not merge your pull requests into the OBSALL main branch by yourself & do not commit (!never!) any changes directly to the main branch!

## Contributing to documentation

Any suggestions on improvements to the OBSALL documentation are always welcome; please, follow same steps as explained in 4.2.

# CHAPTER 5: TROUBLESHOOTING AND FAQ

Frequently asked questions and addressed common issues (that users may encounter when working with OBSALL) are given below.

Q: I am getting an error when trying to clone OBSALL on Lumi, what can I do?

A: Check that you have access to the OBSALL repository at GitLab & check that you typed login and password correctly when accessing repository.

Q: I am getting an error (as an infinite looping) when trying to load all required modules for OBSALL on Lumi, what can I do?

A: Interrupt such loop, logout from your account, login again to your account on Lumi & repeat all previous steps again.

Q: How do I cite OBSALL in my research or publications?

A: When using OBSALL for research purposes, please, cite it and its authors using information provided in the Chapter “REFERENCES AND ACKNOWLEDGMENTS”.

# CHAPTER 6: REFERENCES AND ACKNOWLEDGMENTS

## Citing OBSALL

If you use OBSALL Apps your research or publications, please, cite the Apps using the following format and please include also the following acknowledgements:

@software{obsall,

author       = {Heikki Järvinen, Jouni Räisänen, Lauri Tuppi, Madeleine Ekblom, Alexander Mahura},

title        = {ObsAll: Processing ground-based, radiosounding and satellite observational data for streamed climate modeled data},

year         = {2024},

publisher    = {GitLab},

journal      = {Barcelona Supercomputing Center, Earth Sciences GitLab},

howpublished = {\url{https://earth.bsc.es/gitlab/digital-twins/de_340/obsall/newbranch}},

}

## Acknowledgments

The authors would like to thank many colleagues who have helped in developing, testing, implementing, and maintaining the OBSALL Apps.

The development of OBSALL has been funded by the European Union (EU) through Contract DE_340_CSC – Destination Earth Programme Climate Adaptation Digital Twin (Climate DT). Views and opinions expressed are however those of the authors only and do not necessarily reflect those of EU, the European Commission (EC) or ECMWF. Neither EU and EC nor ECMWF can be held responsible for them.

The Destination Earth is EU funded initiative, and it is implemented by ECMWF (European Centre for Medium-Range Weather Forecasts), ESA (European Space Agency), and EUMETSAT (European Organization for the Exploitation of Meteorological Satellites).

## Especial thanks

Our especial thanks:

- for advise on issues with autosubmit, gsv extraction and other tools/apps, integration in workflow, etc. to BSC colleagues: Miguel Castrillo, Francesc Roura, Iker Gonzalez, Marvin Axness, Aina Gaya i Àvila;

- for constructive comments, remarks, suggestions to reviwers: Steger Christian (DWD), Bruno Kinoshita (BSC), Miguel Castrillo (BSC) & Aina Gaya i Àvila (BSC), Pablo Ortega (BSC), Paolo Davini (CNR), Jost Hardenberg (POLITO), Natalia Nazarova (POLITO);

- for Lumi related advises/help to: Juha Lento (CSC), Devaraju Narayanappa (CSC), Henrik Nortamo (CSC), Sami Salonen (CSC), Roman Nuterman (UCPH), Thomas Geenen (ECMWF); and

- for polytope tool info/examples sharing to: Matthew Griffith, Nils Wedi (ECMWF).